# Aufgaben: IPv6 Adresstypen und Kürzung

IPv6 verwendet die drei folgenden Arten von Adressen.
Übersetzen Sie den folgenden Text sinngemäß ins Deutsche.

Unicast
An identifier for a single interface. Packets will be delivered to the interface identified by that unique address.

Multicast
An identifier for a set of interfaces. Packets will be sent to specific group of interfaces (Example: all hosts, all routers). There are no broadcast addresses in IPv6, their function being superseded by multicast addresses.

Anycast
Anycast is a network addressing and routing methodology in which datagrams from a single sender are routed to the topologically nearest node in a group of potential receivers, though it may be sent to several nodes, all identified by the same destination address.

Übersetzung

### Unicast:

### Multicast:

### Anycast:

RFC 4291
The 128-bit IPv6 address can be abbreviated with the following rules.
Rule one
Leading zeroes within a 16-bit value may be omitted.
For example, the address fe80:0000:0000:0000:0202:b3ff:fe1e:8329 may be written as fe80:0:0:0:202:b3ff:fe1e:8329.

Rule two
A single occurrence of consecutive groups of zeroes within an address may be replaced by a double colon.
For example, the address fe80:0:0:0:202:b3ff:fe1e:8329 becomes fe80::202:b3ff:fe1e:8329.

Verkürzen Sie die folgende IPv6-Adresse nach den Regeln der RFC 4291.
2001:0DB8:0011:0000:0000:0000:0000:0001

2001:DB8:11::1

Tunneling mit 6 to 4 ermöglicht die Verbindung von IPv6-Clouds über ein IPv4-Netzwerk.
Zur Veranschaulichung der Konvertierung zwischen IPv4-Adressen und IPv6-Adressen soll folgendes Beispiel dienen.

Vervollständigen Sie die IPv6-Adresse.
IPv4-Adresse (dezimal)    212.204.101.210
IPv6-Adresse (hexadezimal) 2002:D4CC:65D2::/48
212 -> 1101 0100 -> D4

# Öffentliche IPv6-Adressen und Subnetze

Global unicast addresses are equivalent to public IPv4 addresses. They are globally routable and reachable on the IPv6 Internet. The three high-order bits are set to 001. The address prefix for currently assigned global addresses is 2000::/3. The Global Routing Prefix indicates the global routing prefix for a specific organization’s site. The combination of the three fixed bits and the 45-bit Global Routing Prefix creates a 48-bit site prefix, which is assigned to an individual site of an organization. The Subnet ID is used within an organization’s site to identify subnets. This field is 16 bits long. The organization’s site can use these 16 bits. The Interface ID indicates the interface on a specific subnet within the site. This field is 64 bits long.

Special IPv6 addresses Loopback address The loopback address (0:0:0:0:0:0:0:1 or ::1) is used to identify a loopback interface, enabling a node to send packets to itself. It is equivalent to the IPv4 loopback address of 127.0.0.1. Packets addressed to the loopback address must never be sent on a link or forwarded by a router.

Aufgaben:

a) Geben Sie den Adresstyp an, der als öffentliche IPv6 verwendet wird.

b) Mit der öffentlichen IPv6-Adresse können innerhalb des Unternehmensstandortes Subnetze eingerichtet werden. Ermitteln Sie die Anzahl Subnetze, die maximal gebildet werden können

2001::/48 -> 64 Netzwerkbits – 48 bit Global Routing Prefix = 16 Subnetbits -> 2^16 Subnetze = 65.536 Subnetze

2001:db8:1234::/48 0000 16 / 4 = 4: Schrittlänge/Blocksize

2001:db8:1234:0000:/50
2001:db8:1234:4000:/50
2001:db8:1234:8000:/50
2001:db8:1234:C000:/50

c) Für einen Test verwenden Sie den folgenden "ping"-Befehl auf einem PC:

C:\User>ping ::1

Erläutern Sie diesen Befehl.